Arduino IDE Setup

You can download IDE from this link make sure to download version 1.8.19.

https://www.arduino.cc/en/software

After installing the IDE. Add ESP32 board and required libraries in IDE.

Open Arduino IDE.

Go to Files >> Preferences

Add following link in Additional Board URLs

https://raw.githubusercontent.com/espressif/arduino-esp32/gh-pages/package_esp32_dev_index.json

Click Ok.

Go to Tools >> Boards >> Board Manager

Search for ESP32 and install version 2.0.14

Download libraries from the following GitHub links.

https://github.com/me-no-dev/AsyncTCP

https://arduinojson.org/

https://github.com/me-no-dev/ESPAsyncWebServer

https://github.com/PaulStoffregen/Time

https://github.com/adafruit/RTClib

https://github.com/adafruit/Adafruit_BusIO/tree/master

Go to Sketch >> Include Libraries >> Add .ZIP Libraries.

Navigate to the directory where you downloaded the libraries.

Add them one by one.

After adding libraries now connect ESP32 with PC. Select Port which ESP32 is connected and Board.

Note:

Make sure your Relay is set to active LOW, means jumper shown in image should be connected to LOW and COM pin.

In image it is connected with HIGH and COM.